Methodology of Map:

- Introduction:

The Kas River is a six order tributary of Godavari River. It is the part of Mula basin in hilly zone of Deccan plateau in Maharashtra, India. There is one major dam Kotmara constructed over river and the water stored is utilized for agricultural purposes. The latitude of the site is 19°15'59.52"N and longitude 74° 6'57.09"E. Kas River flows in upper reaches eastward and in lower reaches it turns to north direction and again it flows in eastward direction. The total length of Kas River is 35.25km (from source to confluence). The study area falls in rain shadow zone of Western Ghat and it experiences semiarid climate conditions.

- ISRO data used

DEM (Source-ISRO: C1_DEM_16B_2005-2014_v3_R-1_74E19N_e43c) tile is used from the BHUVAN. DEM refers digital elevation model. It contains elevation details about Study area. These data analyzed by QGIS on the basis of geographical information. It is very important data to analyze the particular region. DEM is the digital representation of the land surface elevation with respect to fixed point.

- Specific steps in GIS:

Step 1. The first thing is to load the project with the DEM data.

Figure 1. DEM loaded in the QGIS workspace

Step2:  to create virtual DEM data

We need to obtain the virtual DEM data which gives the value at a highest range/accuracy. For that we used the tool of QGIS –Raster-Miscellaneous- Build virtual raster.

Figure 2: Virtual DEM Data conversion

Step3: To prepare subset for our area of interest

As our DEM tile is quite large and it takes time process n software therefore we need to prepare the subset only for our area of interest.

Figure 3: DEM Subset

Figure 4 : DEM Subset output

Step 4: Fill the sinks

The DEM data contains lots of gap interms of point so SAGA algorithm used to fill it. We have used the toolbox ‘fill sinks (wang & liu)’ to complete this step.

Figure 5: DEM_Fill

Figure 6: Layer Visualization of Filled DEM

Step 5: to define the Strahler Order

We have obtained the strahler order with the help of toolbox SAGA- Terrain Analysis-Channel-Strahler Order

Figure 7: Tool box of Strahler’s Order

Figure 8: Output of Strahler’s tool

Figure 9: Selection of Stream up to Strahler’s order 5.

Step 6: To define channel network and drainage basin:

We have obtained the channel network and different basins present for a selected DEM data with help of SAGA-Terrain analysis-channels-channel network and drainage basin.

Figure 10: Channel network and drainage basin toolbox

Figure 11: Output of channel network and drainage basin tool

Step 7: to find out the respective co-ordinate for which upstream catchment area need to find.

We have obtained the catchment area for the stream Kas River with the help of background GIS map layer and co-ordinate capture tool

Figure 12:  Co-ordinate capture tool to find out the area upstream of it.

Step 8: Upslope area /Catchment Area calculation

We have obtained the catchment area upstream with the help of tool SAGA-Terrain Analysis-Hydrology-Upslope Area.

Figure 13: Catchment Area selection with the help of Upslope area tool

Step 9: Convert given output of upslope area into vector format and again saved only the catchment polygon as a separate shape file

Figure 14: Catchment Area Calculation in QGIS

- Application and use of these maps.

- To know the drainage pattern of the area.

- To do the Dam assessment study

- To know the hydrological features and stream patterns, catchment area etc. those are the inputs for any water resource engineering work in the area.

- Ecological Survey of the region

- Tourism purpose-can help to enhance the agricultural –rural-tribal developments

- To make awareness about the importance of rivers and protection of the natural features e.g. forests, water resource.

- Opportunities to develop the rural part with the help of proper utilization of water resources.

- Final Output: